Echo Shapes and Reflections LED 9.0 Multiple Pendants

Using Single 0-degree ies file as base file to config/convert top & bottom or sided by side configurations for Echo Shapes

Step by step procedure

- Pull in IES single 0° tilt file from SPI website [SHAPES SINGLE 0 DEGREE.ies]

  - Label:  SHAPES SINGLE 0 DEGREE

  - Description: L63W (1M)

  - LLF: .9

    - See Below Table

  - Click into Symbols:  Choose Box Up & [Symbol Scaling:  X = 0.23,  Y = 0.857,  Z = 0.522]

    - See table below

- Locate the L63W (1M) in plan anywhere outside the building or room.

- After rotating the modules to the correct angle required, next step is to array the module to over-under LED configuration – See below

EEW12091-L127W

Refer to AGi file to import

- Skip to Aiming Individual Units

- For Over-under, array luminaire X direction spacing to equal .3540 ft [4.25” true offset]

- Follow next steps to creating pendant multiples…

- Double Cluster – Use SPI online web Spec. Sheet for fixture dimension spacings

    - Mirror  all optics

    - Using Spec sheet (OAL = 37.3”, AGI scaling for 37.3 is 3.11”) and move mirrored optics to 3.11”   * See below

- Create fixture group to assign catalog ID [In the Model Toolkit/Luminaire section, click “Create luminaire Group”]

- Select luminaire (Window box around the symbols]

- Click desired insertion point

- Assign Catalog ID [Ex. EIP12129] & Description as shown below

- Triple Cluster – Use SPI online web Spec. Sheet for fixture dimension spacings

    - Refer to Step 7 first to create a two optic fixture group first before array

    - Polar Array grouped fixture created *see below

    - Using Spec sheet (OAW = 43.2”, AGI scaling for 43.2 is 3.60”) and move optics to get all three sides to match 3.60”   * See below

    - Explode  all three grouped units shown above.

    - Refer to step 7 to group all three symbolled optics to create EIP12128

- Quad Cluster – Use SPI online web Spec. Sheet for fixture dimension spacings

    - Refer to Step 7 first to create a two optic fixture group first before array

    - Polar Array grouped fixture created *see below

    - Using Spec sheet (OAW = 43.3” & OAL = 43.3”, AGI scaling for 43.3 is 3.608”) and move optics to get all fours sides to match 3.608”   * See below

    - Explode  all four grouped units as shown above.

    - Refer to step 7 to re-create new group with all four symbolled optics to create EIP12127

AIMING INDIVIDUAL UNITS

- Click Add/Redefine tab to create the group luminaire.

Note:  In above “Define Luminaire” section, to change tilt, highlight over the (-35) and change to the degree desired.

  - Note: Option for tilting are as follows:

    - Negative tilt = positive aiming

    - Positive tilt = negative aiming

      - See example below

Positive Tilt                   Negative Tilt